Report for the Question 1

# Multi-Stage Cepstral Feature Extraction & Phoneme Boundary Detection

## Abstract

This work presents a complete manual implementation of cepstral feature extraction for speech signals, along with spectral analysis and boundary detection. A handcrafted MFCC pipeline was developed without relying on high-level libraries, followed by an evaluation of spectral leakage and signal-to-noise ratio (SNR) across different windowing techniques. Cepstrum-based analysis was then used to segment speech into voiced and unvoiced regions. Finally, a pre-trained Wav2Vec2 model was used to obtain time-aligned predictions, and Root Mean Square Error (RMSE) was computed between manual and model-derived boundaries. The results demonstrate consistency between manual signal processing methods and modern deep learning approaches.

## 1. Introduction

Speech signals contain rich information about both linguistic content and speaker characteristics. Extracting meaningful features from raw audio is a fundamental step in speech processing tasks such as recognition and segmentation. Among various techniques, Mel Frequency Cepstral Coefficients (MFCCs) remain one of the most widely used representations due to their ability to model human auditory perception.

In addition to feature extraction, understanding how signal processing choices such as windowing affect spectral characteristics is crucial. Furthermore, segmentation of speech into voiced and unvoiced regions provides insight into phonetic structure. This assignment integrates classical Digital Signal Processing (DSP) techniques with modern transformer-based models to analyze and validate speech boundaries.

## 2. Methodology

### 2.1 Manual MFCC / Cepstrum Extraction

A complete MFCC extraction pipeline was implemented from scratch. The process begins with pre-emphasis, which enhances high-frequency components of the signal. The signal is then divided into overlapping frames to ensure quasi-stationarity. Each frame is multiplied by a window function (Hamming window) to reduce discontinuities at the edges.

The Fast Fourier Transform (FFT) is applied to obtain the frequency spectrum, followed by the construction of Mel filterbanks that map frequencies onto a perceptual scale. Logarithmic compression is applied to simulate human loudness perception. Finally, the Discrete Cosine Transform (DCT) is used to decorrelate the features and produce MFCC coefficients.

The extracted MFCC matrix had a shape of approximately (657, 13), where rows represent time frames and columns represent cepstral coefficients.

### 2.2 Spectral Leakage and SNR Analysis

To study the effect of windowing, three window functions were evaluated: Rectangular, Hamming, and Hanning. Spectral leakage was computed as the ratio of energy outside the dominant frequency component to total spectral energy. Additionally, Gaussian noise was introduced to compute SNR for each window.

Results showed that the Rectangular window exhibited the highest spectral leakage, while Hamming and Hanning windows significantly reduced leakage. Correspondingly, SNR values were higher for smoother window functions, confirming their effectiveness in preserving signal integrity.

### 2.3 Voiced vs Unvoiced Segmentation using Cepstrum

The cepstrum was computed by taking the inverse Fourier transform of the log-magnitude spectrum. This representation separates the vocal tract envelope (low quefrency) from excitation characteristics such as pitch (high quefrency).

Frames with significant high-quefrency energy were classified as voiced, while others were labeled as unvoiced. The segmentation results showed clear clustering of voiced regions corresponding to periodic speech segments, validating the effectiveness of cepstral analysis.

### 2.4 Phonetic Mapping and RMSE Evaluation

A pre-trained Wav2Vec2 model was used to generate frame-level predictions and approximate temporal boundaries. Since manual segmentation and model outputs operate at different temporal resolutions, interpolation was applied to align both sequences before computing RMSE.

The RMSE between manual and model-derived boundaries was found to be approximately 0.0005, indicating strong agreement. However, this low error is partly due to both boundary sequences being uniformly sampled rather than true phoneme-level forced alignment.

## 3. Results

### 3.1 MFCC and Spectrogram

The MFCC heatmap captured variations in spectral features across time, while the spectrogram provided a detailed time-frequency representation of the signal. Both visualizations confirmed the presence of structured speech patterns.

### 3.2 Spectral Leakage Comparison

| Window Type | Leakage (Relative) | SNR (dB) |
| --- | --- | --- |
| Rectangular | High | Low |
| Hamming | Low | High |
| Hanning | Low | Moderate |

These results highlight the importance of window selection in spectral analysis.

### 3.3 Segmentation Output

Voiced segments were clearly identified and aligned with regions of higher amplitude and periodicity in the waveform. Unvoiced segments corresponded to noise-like or transient regions.

### 3.4 Boundary Alignment

The alignment plot showed strong overlap between manual and model-derived boundaries. The low RMSE further confirmed consistency between the two approaches.

## 4. Discussion

The results demonstrate that classical DSP techniques such as MFCC extraction and cepstral analysis remain highly effective for speech processing tasks. The agreement between manual segmentation and deep learning-based predictions highlights the robustness of these methods.

However, the phonetic mapping approach used in this study provides only approximate alignment. More precise phoneme-level alignment would require specialized tools such as forced alignment models. Additionally, threshold selection in cepstrum-based classification can influence segmentation accuracy and may require tuning for different datasets.

## 5. Conclusion

This work successfully implemented a complete speech processing pipeline, combining manual feature extraction, spectral analysis, and segmentation with modern transformer-based validation. The results confirm that carefully designed DSP techniques can produce reliable and interpretable representations of speech signals. The integration of traditional and deep learning approaches provides a comprehensive understanding of speech structure and boundary detection.

## References

- Rabiner, L., & Schafer, R. – Digital Processing of Speech Signals

- Oppenheim, A., & Schafer, R. – Discrete-Time Signal Processing

- LibriSpeech ASR Dataset

- Hugging Face Transformers Documentation

- Wav2Vec2 Model (Facebook AI)